These are the burnfiles for OS96 for a 64kb-EPROM (27512). It's possible to

use only the low-part (27256 or 2*27128-EPROMs). But then OS96 doesn't trans-

fer values from setup.

For control the A15-line you will need the 256kb-RAM-expansion and a logic-

device. For informations see "hardware.txt".

Features of OS96:

- Datalink to PC

- setup-utility with sRAM-Card or -expansion

- Printerdriver for pinters at User- or printerport

- fast OPEN1,8,15-command in BASIC and monitor

- complete hardwarecheck

Implemented are:

pc-link. You need a special parallel-cable (see hardware.txt for informations)

and the pc-program 1551.exe.

You can use the pc same as a floppy. That means you can use the commands LOAD/

SAVE and OPEN. The pc can be connected with the following ports. You select

it in the setup. This value will be copied to adress $bb:

$00 no PC

$01 PC at Userport

$30 PC at expansionuserport adress $fd60

$32 PC at expansionuserport adress $fd64

$34 PC at expansionuserport adress $fd68

$36 PC at expansionuserport adress $fd6c

The pc has a higher priority than a serial device, but a lower than a 1551.

OS96 doesn't made a timeout if no pc is available. This is good for multitas-

king systems on pc. But you can break the waiting for a pc if you press RUN/

STOP. This creates a device not present error and sets the adress $bb to

zero. For use the pc now you must reset the value in $bb or press reset-key

(if you have 64k-version with setup).

Printer-driver for printer with centronics(parallel)-port: You can use this

printers same as a serial connected printer. The following ports are supported.

It will be activated in adress $07fc:

$00: serial printer

$01: special printerport (with standard-printercable)

$02: userport (Userport-Centronicscable)

$60: extented-userport adress $fd60

$64: extended-userport adress $fd64

$68: extended-userport adress $fd68

$6c: extended-userport adress $fd6c

The driver-routine is very compatible. You can directly print a directory on

printer (use filenumber <>1, because the 1 is used by the directory-routine).

But there is a little problem with script/plus and the (normal) userport.

Please use the internal driver. 'Cause the channel-principle there isn't a

timeout-flag in OS96. The program waits eternal for the printer if it isn't

ready.

Setup-Routine:

This routine is implemented in the high-part of OS96. The actual version co-

pies only the values for printer and pc from sRAM. It doesn't exist a setup-

program. You must transfer the values to sRAM manually with following steps.

for the internal expansion:

1.) switch off write-protect

2.) >fd17 fe

3.) >7f8 80

4.) >7f40 53 45 54 55 50     ;setup

5.) >7f45 xx                 ;value for pc-link

6.) >7f46 xx                 ;value for printer

7.) >fd17 ff

8.) switch on write-protect

for the external sRAM-card, it's important, that a modul

is in area 0!!!

1.) switch off write-protect

2.) >fd17 7f

3.) >fdff 00

4.) >7f8 0a

5.) m8000  -> search for "CBM" from $8007-$8009, if yes

then notice "bank a", else "bank f"

6.) if bank a: >fdff 40

if bank f: >fdff 80

7.) >7f40 53 45 54 55 50  ;"setup"

8.) >7f45 xx              ;value for pc-link

9.) >7f46 xx              ;value for printer

7.) >fdff 00

8.) switch on write protect

You can check all inputs. Activate the internal or external

sRAM-expansion, switch to standard-area, go in monito

1.) >7f8 0x     (for x set the bank 5, a or f)

2.) mff40

Now you should see the "setup"-mark and the values for the

pc-link and printer

OS96 expands the "7f8"-function

If you remember you can use the value $80 in adress $07f8 to get access to the

RAM over $8000 for the monitor-commands. OS96 expands this function. Here are

the new values:

$0x: access to the selected ROM-bank, standard is $00 (Basic/Kernal) - same as

before

$40: access to OS96 high

$80: access to RAM over $8000 (same as before)

version-number:

You can see the actual version in RAM-adresses $fcef/fcf0 in low/high-

format.

Compatibility: The further 32kB-version had problems with some programs. I had

deleted to many tape-routines. Some games and packers wich doesn't work with

OS92.3, doesn't work with OS96 too. I will search the problem. The trainer of

the game "TOM" runs now with OS96 without problems.

Solder/Synergy, c/o Christian Schffner, Pfarrplan 1, D-39397 Schwanebeck,

Germany

Plus/4-BBS from 18 CET: +49 39424/8595, V34+, ANSI

e-mail: solder@t-online.de

WWW: http://home.t-online.de/home/solder